# MIPS BASIC INSTRUCTIONS

- abs.d $f2,$f4 Floating point absolute value double precision : Set $f2 to absolute value of $f4, double precision

- abs.s $f0,$f1 Floating point absolute value single precision : Set $f0 to absolute value of $f1, single precision

- add $t1,$t2,$t3 Addition with overflow : set $t1 to ($t2 plus $t3)

- add.d $f2,$f4,$f6 Floating point addition double precision : Set $f2 to double-precision floating point value of $f4 plus $f6

- add.s $f0,$f1,$f3 Floating point addition single precision : Set $f0 to single-precision floating point value of $f1 plus $f3

- addi $t1,$t2,-100 Addition immediate with overflow : set $t1 to ($t2 plus signed 16-bit immediate)

- addiu $t1,$t2,-100 Addition immediate unsigned without overflow : set $t1 to ($t2 plus signed 16-bit immediate), no overflow

- addu $t1,$t2,$t3 Addition unsigned without overflow : set $t1 to ($t2 plus $t3), no overflow

- and $t1,$t2,$t3 Bitwise AND : Set $t1 to bitwise AND of $t2 and $t3

- andi $t1,$t2,100 Bitwise AND immediate : Set $t1 to bitwise AND of $t2 and zero-extended 16-bit immediate

- bc1f 1,label Branch if specified FP condition flag false (BC1F, not BCLF) : If Coprocessor 1 condition flag specified by immediate is false (zero) then branch to statement at label's address

- bc1f label Branch if FP condition flag 0 false (BC1F, not BCLF) : If Coprocessor 1 condition flag 0 is false (zero) then branch to statement at label's address

- bc1t 1,label Branch if specified FP condition flag true (BC1T, not BCLT) : If Coprocessor 1 condition flag specified by immediate is true (one) then branch to statement at label's address

- bc1t label Branch if FP condition flag 0 true (BC1T, not BCLT) : If Coprocessor 1 condition flag 0 is true (one) then branch to statement at label's address

- beq $t1,$t2,label Branch if equal : Branch to statement at label's address if $t1 and $t2 are equal

- bgez $t1,label Branch if greater than or equal to zero : Branch to statement at label's address if $t1 is greater than or equal to zero

- bgezal $t1,label Branch if greater then or equal to zero and link : If $t1 is greater than or equal to zero, then set $ra to the Program Counter and branch to statement at label's address

- bgtz $t1,label Branch if greater than zero : Branch to statement at label's address if $t1 is greater than zero

- blez $t1,label Branch if less than or equal to zero : Branch to statement at label's address if $t1 is less than or equal to zero

- bltz $t1,label Branch if less than zero : Branch to statement at label's address if $t1 is less than zero

- bltzal $t1,label Branch if less than zero and link : If $t1 is less than or equal to zero, then set $ra to the Program Counter and branch to statement at label's address

- bne $t1,$t2,label Branch if not equal : Branch to statement at label's address if $t1 and $t2 are not equal

- break Break execution : Terminate program execution with exception

- break 100 Break execution with code : Terminate program execution with specified exception code

- c.eq.d $f2,$f4 Compare equal double precision : If $f2 is equal to $f4 (double-precision), set Coprocessor 1 condition flag 0 true else set it false

- c.eq.d 1,$f2,$f4 Compare equal double precision : If $f2 is equal to $f4 (double-precision), set Coprocessor 1 condition flag specified by immediate to true else set it to false

- c.eq.s $f0,$f1 Compare equal single precision : If $f0 is equal to $f1, set Coprocessor 1 condition flag 0 true else set it false

- c.eq.s 1,$f0,$f1 Compare equal single precision : If $f0 is equal to $f1, set Coprocessor 1 condition flag specied by immediate to true else set it to false

- c.le.d $f2,$f4 Compare less or equal double precision : If $f2 is less than or equal to $f4 (double-precision), set Coprocessor 1 condition flag 0 true else set it false

- c.le.d 1,$f2,$f4 Compare less or equal double precision : If $f2 is less than or equal to $f4 (double-precision), set Coprocessor 1 condition flag specfied by immediate true else set it false

- c.le.s $f0,$f1 Compare less or equal single precision : If $f0 is less than or equal to $f1, set Coprocessor 1 condition flag 0 true else set it false

- c.le.s 1,$f0,$f1 Compare less or equal single precision : If $f0 is less than or equal to $f1, set Coprocessor 1 condition flag specified by immediate to true else set it to false

- c.lt.d $f2,$f4 Compare less than double precision : If $f2 is less than $f4 (double-precision), set Coprocessor 1 condition flag 0 true else set it false

- c.lt.d 1,$f2,$f4 Compare less than double precision : If $f2 is less than $f4 (double-precision), set Coprocessor 1 condition flag specified by immediate to true else set it to false

- c.lt.s $f0,$f1 Compare less than single precision : If $f0 is less than $f1, set Coprocessor 1 condition flag 0 true else set it false

- c.lt.s 1,$f0,$f1 Compare less than single precision : If $f0 is less than $f1, set Coprocessor 1 condition flag specified by immediate to true else set it to false

- ceil.w.d $f1,$f2 Ceiling double precision to word : Set $f1 to 32-bit integer ceiling of double-precision float in $f2

- ceil.w.s $f0,$f1 Ceiling single precision to word : Set $f0 to 32-bit integer ceiling of single-precision float in $f1

- clo $t1,$t2 Count number of leading ones : Set $t1 to the count of leading one bits in $t2 starting at most significant bit position

- clz $t1,$t2 Count number of leading zeroes : Set $t1 to the count of leading zero bits in $t2 starting at most significant bit positio

- cvt.d.s $f2,$f1 Convert from single precision to double precision : Set $f2 to double precision equivalent of single precision value in $f1

- cvt.d.w $f2,$f1 Convert from word to double precision : Set $f2 to double precision equivalent of 32-bit integer value in $f1

- cvt.s.d $f1,$f2 Convert from double precision to single precision : Set $f1 to single precision equivalent of double precision value in $f2

- cvt.s.w $f0,$f1 Convert from word to single precision : Set $f0 to single precision equivalent of 32-bit integer value in $f2

- cvt.w.d $f1,$f2 Convert from double precision to word : Set $f1 to 32-bit integer equivalent of double precision value in $f2

- cvt.w.s $f0,$f1 Convert from single precision to word : Set $f0 to 32-bit integer equivalent of single precision value in $f1

- div $t1,$t2 Division with overflow : Divide $t1 by $t2 then set LO to quotient and HI to remainder (use mfhi to access HI, mflo to access LO)

- div.d $f2,$f4,$f6 Floating point division double precision : Set $f2 to double-precision floating point value of $f4 divided by $f6

- div.s $f0,$f1,$f3 Floating point division single precision : Set $f0 to single-precision floating point value of $f1 divided by $f3

- divu $t1,$t2 Division unsigned without overflow : Divide unsigned $t1 by $t2 then set LO to quotient and HI to remainder (use mfhi to access HI, mflo to access LO)

- eret Exception return : Set Program Counter to Coprocessor 0 EPC register value, set Coprocessor Status register bit 1 (exception level) to zero

- floor.w.d $f1,$f2 Floor double precision to word : Set $f1 to 32-bit integer floor of double-precision float in $f2

- floor.w.s $f0,$f1 Floor single precision to word : Set $f0 to 32-bit integer floor of single-precision float in $f1

- j target Jump unconditionally : Jump to statement at target address

- jal target Jump and link : Set $ra to Program Counter (return address) then jump to statement at target address

- jalr $t1 Jump and link register : Set $ra to Program Counter (return address) then jump to statement whose address is in $t1

- jalr $t1,$t2 Jump and link register : Set $t1 to Program Counter (return address) then jump to statement whose address is in $t2

- jr $t1 Jump register unconditionally : Jump to statement whose address is in $t1

- lb $t1,-100($t2) Load byte : Set $t1 to sign-extended 8-bit value from effective memory byte address

- lbu $t1,-100($t2) Load byte unsigned : Set $t1 to zero-extended 8-bit value from effective memory byte address

- ldc1 $f2,-100($t2) Load double word Coprocessor 1 (FPU)) : Set $f2 to 64-bit value from effective memory doubleword address

- lh $t1,-100($t2) Load halfword : Set $t1 to sign-extended 16-bit value from effective memory halfword address

- lhu $t1,-100($t2) Load halfword unsigned : Set $t1 to zero-extended 16-bit value from effective memory halfword address

- ll $t1,-100($t2) Load linked : Paired with Store Conditional (sc) to perform atomic read-modify-write. Treated as equivalent to Load Word (lw) because MARS does not simulate multiple processors.

- lui $t1,100 Load upper immediate : Set high-order 16 bits of $t1 to 16-bit immediate and low-order 16 bits to 0

- lw $t1,-100($t2) Load word : Set $t1 to contents of effective memory word address

- lwc1 $f1,-100($t2) Load word into Coprocessor 1 (FPU) : Set $f1 to 32-bit value from effective memory word address

- lwl $t1,-100($t2) Load word left : Load from 1 to 4 bytes left-justified into $t1, starting with effective memory byte address and continuing through the low-order byte of its word

- lwr $t1,-100($t2) Load word right : Load from 1 to 4 bytes right-justified into $t1, starting with effective memory byte address and continuing through the high-order byte of its word

- madd $t1,$t2 Multiply add : Multiply $t1 by $t2 then increment HI by high-order 32 bits of product, increment LO by low-order 32 bits of product (use mfhi to access HI, mflo to access LO)

- maddu $t1,$t2 Multiply add unsigned : Multiply $t1 by $t2 then increment HI by high-order 32 bits of product, increment LO by low-order 32 bits of product, unsigned (use mfhi to access HI, mflo to access LO)

- mfc0 $t1,$8 Move from Coprocessor 0 : Set $t1 to the value stored in Coprocessor 0 register $8

- mfc1 $t1,$f1 Move from Coprocessor 1 (FPU) : Set $t1 to value in Coprocessor 1 register $f1

- mfhi $t1 Move from HI register : Set $t1 to contents of HI (see multiply and divide operations)

- mflo $t1 Move from LO register : Set $t1 to contents of LO (see multiply and divide operations)

- mov.d $f2,$f4 Move floating point double precision : Set double precision $f2 to double precision value in $f4

- mov.s $f0,$f1 Move floating point single precision : Set single precision $f0 to single precision value in $f1

- movf $t1,$t2 Move if FP condition flag 0 false : Set $t1 to $t2 if FPU (Coprocessor 1) condition flag 0 is false (zero)

- movf $t1,$t2,1 Move if specified FP condition flag false : Set $t1 to $t2 if FPU (Coprocessor 1) condition flag specified by the immediate is false (zero)

- movf.d $f2,$f4 Move floating point double precision : If condition flag 0 false, set double precision $f2 to double precision value in $f4

- movf.d $f2,$f4,1 Move floating point double precision : If condition flag specified by immediate is false, set double precision $f2 to double precision value in $f4

- movf.s $f0,$f1 Move floating point single precision : If condition flag 0 is false, set single precision $f0 to single precision value in $f1

- movf.s $f0,$f1,1 Move floating point single precision : If condition flag specified by immediate is false, set single precision $f0 to single precision value in $f1e

- movn $t1,$t2,$t3 Move conditional not zero : Set $t1 to $t2 if $t3 is not zero

- movn.d $f2,$f4,$t3 Move floating point double precision : If $t3 is not zero, set double precision $f2 to double precision value in $f4

- movn.s $f0,$f1,$t3 Move floating point single precision : If $t3 is not zero, set single precision $f0 to single precision value in $f1

- movt $t1,$t2 Move if FP condition flag 0 true : Set $t1 to $t2 if FPU (Coprocessor 1) condition flag 0 is true (one)

- movt $t1,$t2,1 Move if specfied FP condition flag true : Set $t1 to $t2 if FPU (Coprocessor 1) condition flag specified by the immediate is true (one)

- movt.d $f2,$f4 Move floating point double precision : If condition flag 0 true, set double precision $f2 to double precision value in $f4

- movt.d $f2,$f4,1 Move floating point double precision : If condition flag specified by immediate is true, set double precision $f2 to double precision value in $f4e

- movt.s $f0,$f1 Move floating point single precision : If condition flag 0 is true, set single precision $f0 to single precision value in $f1e

- movt.s $f0,$f1,1 Move floating point single precision : If condition flag specified by immediate is true, set single precision $f0 to single precision value in $f1e

- movz $t1,$t2,$t3 Move conditional zero : Set $t1 to $t2 if $t3 is zero

- movz.d $f2,$f4,$t3 Move floating point double precision : If $t3 is zero, set double precision $f2 to double precision value in $f4

- movz.s $f0,$f1,$t3 Move floating point single precision : If $t3 is zero, set single precision $f0 to single precision value in $f1

- msub $t1,$t2 Multiply subtract : Multiply $t1 by $t2 then decrement HI by high-order 32 bits of product, decrement LO by low-order 32 bits of product (use mfhi to access HI, mflo to access LO)

- msubu $t1,$t2 Multiply subtract unsigned : Multiply $t1 by $t2 then decrement HI by high-order 32 bits of product, decement LO by low-order 32 bits of product, unsigned (use mfhi to access HI, mflo to access LO)

- mtc0 $t1,$8 Move to Coprocessor 0 : Set Coprocessor 0 register $8 to value stored in $t1

- mtc1 $t1,$f1 Move to Coprocessor 1 (FPU) : Set Coprocessor 1 register $f1 to value in $t1

- mthi $t1 Move to HI registerr : Set HI to contents of $t1 (see multiply and divide operations)

- mtlo $t1 Move to LO register : Set LO to contents of $t1 (see multiply and divide operations)

- mul $t1,$t2,$t3 Multiplication without overflow : Set HI to high-order 32 bits, LO and $t1 to low-order 32 bits of the product of $t2 and $t3 (use mfhi to access HI, mflo to access LO)

- mul.d $f2,$f4,$f6 Floating point multiplication double precision : Set $f2 to double-precision floating point value of $f4 times $f6

- mul.s $f0,$f1,$f3 Floating point multiplication single precision : Set $f0 to single-precision floating point value of $f1 times $f3

- mult $t1,$t2 Multiplication : Set hi to high-order 32 bits, lo to low-order 32 bits of the product of $t1 and $t2 (use mfhi to access hi, mflo to access lo)

- multu $t1,$t2 Multiplication unsigned : Set HI to high-order 32 bits, LO to low-order 32 bits of the product of unsigned $t1 and $t2 (use mfhi to access HI, mflo to access LO)

- neg.d $f2,$f4 Floating point negate double precision : Set double precision $f2 to negation of double precision value in $f4

- neg.s $f0,$f1 Floating point negate single precision : Set single precision $f0 to negation of single precision value in $f1

- nop Null operation : machine code is all zeroes

- nor $t1,$t2,$t3 Bitwise NOR : Set $t1 to bitwise NOR of $t2 and $t3

- or $t1,$t2,$t3 Bitwise OR : Set $t1 to bitwise OR of $t2 and $t3

- ori $t1,$t2,100 Bitwise OR immediate : Set $t1 to bitwise OR of $t2 and zero-extended 16-bit immediate

- round.w.d $f1,$f2 Round double precision to word : Set $f1 to 32-bit integer round of double-precision float in $f2

- round.w.s $f0,$f1 Round single precision to word : Set $f0 to 32-bit integer round of single-precision float in $f1

- sb $t1,-100($t2) Store byte : Store the low-order 8 bits of $t1 into the effective memory byte address

- sc $t1,-100($t2) Store conditional : Paired with Load Linked (ll) to perform atomic read-modify-write. Stores $t1 value into effective address, then sets $t1 to 1 for success. Always succeeds because MARS does not simulate multiple processors.

- sdc1 $f2,-100($t2) Store double word from Coprocessor 1 (FPU)) : Store 64 bit value in $f2 to effective memory doubleword address

- sh $t1,-100($t2) Store halfword : Store the low-order 16 bits of $t1 into the effective memory halfword address

- sll $t1,$t2,10 Shift left logical : Set $t1 to result of shifting $t2 left by number of bits specified by immediate

- sllv $t1,$t2,$t3 Shift left logical variable : Set $t1 to result of shifting $t2 left by number of bits specified by value in low-order 5 bits of $t3

- slt $t1,$t2,$t3 Set less than : If $t2 is less than $t3, then set $t1 to 1 else set $t1 to 0

- slti $t1,$t2,-100 Set less than immediate : If $t2 is less than sign-extended 16-bit immediate, then set $t1 to 1 else set $t1 to 0

- sltiu $t1,$t2,-100 Set less than immediate unsigned : If $t2 is less than sign-extended 16-bit immediate using unsigned comparison, then set $t1 to 1 else set $t1 to 0

- sltu $t1,$t2,$t3 Set less than unsigned : If $t2 is less than $t3 using unsigned comparision, then set $t1 to 1 else set $t1 to 0

- sqrt.d $f2,$f4 Square root double precision : Set $f2 to double-precision floating point square root of $f4

- sqrt.s $f0,$f1 Square root single precision : Set $f0 to single-precision floating point square root of $f1

- sra $t1,$t2,10 Shift right arithmetic : Set $t1 to result of sign-extended shifting $t2 right by number of bits specified by immediate

- srav $t1,$t2,$t3 Shift right arithmetic variable : Set $t1 to result of sign-extended shifting $t2 right by number of bits specified by value in low-order 5 bits of $t3

- srl $t1,$t2,10 Shift right logical : Set $t1 to result of shifting $t2 right by number of bits specified by immediate

- srlv $t1,$t2,$t3 Shift right logical variable : Set $t1 to result of shifting $t2 right by number of bits specified by value in low-order 5 bits of $t3

- sub $t1,$t2,$t3 Subtraction with overflow : set $t1 to ($t2 minus $t3)

- sub.d $f2,$f4,$f6 Floating point subtraction double precision : Set $f2 to double-precision floating point value of $f4 minus $f6

- sub.s $f0,$f1,$f3 Floating point subtraction single precision : Set $f0 to single-precision floating point value of $f1 minus $f3

- subu $t1,$t2,$t3 Subtraction unsigned without overflow : set $t1 to ($t2 minus $t3), no overflow

- sw $t1,-100($t2) Store word : Store contents of $t1 into effective memory word address

- swc1 $f1,-100($t2) Store word from Coprocesor 1 (FPU) : Store 32 bit value in $f1 to effective memory word address

- swl $t1,-100($t2) Store word left : Store high-order 1 to 4 bytes of $t1 into memory, starting with effective byte address and continuing through the low-order byte of its word

- swr $t1,-100($t2) Store word right : Store low-order 1 to 4 bytes of $t1 into memory, starting with high-order byte of word containing effective byte address and continuing through that byte address

- syscall Issue a system call : Execute the system call specified by value in $v0

- teq $t1,$t2 Trap if equal : Trap if $t1 is equal to $t2

- teqi $t1,-100 Trap if equal to immediate : Trap if $t1 is equal to sign-extended 16 bit immediate

- tge $t1,$t2 Trap if greater or equal : Trap if $t1 is greater than or equal to $t2

- tgei $t1,-100 Trap if greater than or equal to immediate : Trap if $t1 greater than or equal to sign-extended 16 bit immediate

- tgeiu $t1,-100 Trap if greater or equal to immediate unsigned : Trap if $t1 greater than or equal to sign-extended 16 bit immediate, unsigned comparison

- tgeu $t1,$t2 Trap if greater or equal unsigned : Trap if $t1 is greater than or equal to $t2 using unsigned comparision

- tlt $t1,$t2 Trap if less than: Trap if $t1 less than $t2

- tlti $t1,-100 Trap if less than immediate : Trap if $t1 less than sign-extended 16-bit immediate

- tltiu $t1,-100 Trap if less than immediate unsigned : Trap if $t1 less than sign-extended 16-bit immediate, unsigned comparison

- tltu $t1,$t2 Trap if less than unsigned : Trap if $t1 less than $t2, unsigned comparison

- tne $t1,$t2 Trap if not equal : Trap if $t1 is not equal to $t2

- tnei $t1,-100 Trap if not equal to immediate : Trap if $t1 is not equal to sign-extended 16 bit immediate

- trunc.w.d $f1,$f2 Truncate double precision to word : Set $f1 to 32-bit integer truncation of double-precision float in $f2

- trunc.w.s $f0,$f1 Truncate single precision to word : Set $f0 to 32-bit integer truncation of single-precision float in $f1

- xor $t1,$t2,$t3 Bitwise XOR (exclusive OR) : Set $t1 to bitwise XOR of $t2 and $t3

- xori $t1,$t2,100 Bitwise XOR immediate : Set $t1 to bitwise XOR of $t2 and zero-extended 16-bit immediate